SMA5505 - Applied Parallel Computing

Simulation of a Parallel Computing Routing Algorithm for IP/DWDM network

Mid-Term Report

Submitted to Computer Science of SMA

By         Ma Keng Teck (smap1079)

Yu Jia                (smap1062)

Ou Hanyan       ( smap1050)

15 April 2002

Content Table:

Algorithm Background	3

1. DWDM and GMPLS	3

2. Why compute 2 disjoint paths for each request?	3

3. Why this parallel implementation will be interesting and practical?	3

4. Goal	3

Network Scenario	3

Algorithm	4

Our progress	7

* The problems we found so far:	7

Reference:	8

# Algorithm Background

## 1. DWDM and GMPLS

In recent years, advances in dense wavelength-division multiplexing (DWDM) technology have enabled the deployment of systems that are capable of providing large amounts of bandwidth (10Tb/s/optical fiber) in optical network. In addition, The Internet Engineering Task force (IETF) is currently working on generalized multiprotocol label switching (GMPLS), a generalized control framework for establishing various types of connections, including packet switching, wavelength switching and space switching etc.

## 2. Why compute 2 disjoint paths for each request?

As we all know, in optical network, a single fiber cut is frequent and will cause severe data loss in different lightpaths that traverse this failed fiber. We hope to provide end-to-end protection for traffic by allocating backup lightpath through which data can be routed in case of primary lightpath failure.

## 3. Why this parallel implementation will be interesting and practical?

The implementation of this algorithm is unique, because current IP/DWDM routing algorithms and implementations are all based on centralized network model. They assume the algorithm knows all the bandwidth, wavelength information on each link in the network. This is seldom practical in real world network.  In our model, we apply distributed network, each optical node (OXC, optical crossconnect) knows a subnet of IP network. The optical node should communicate with neighbors to get the residual bandwidth information and wavelength usage on optical links, and update the residual bandwidth on each link after a request processing.

## 4. Goal

Bandwidth efficiency, expeditious path computation are desirable features for the design of the algorithms.

# Network Scenario

Our test network model is very similar with 15-node NSF network used in U.S..(see above graph). A more complicated network model can be found at:

http://www.ucalgary.ca/~seto/inter/nsfnet.htm

# Algorithm

- Each processor on hydra cluster acts as an OXC in optical network. If we use 16 processors, we can simulate a 16 node optical network (just like NSFNET).

- Each processor also manages an IP sub-network, and knows the topology of the IP sub-network. Since our focus is on optical network routing, so we simplify the IP network implementation.

- For each request from host A to host B,

  - If host A and host B are in the same subnet, we can use ILP (Integer Linear Programming) solver to find two optical disjoint paths.

  - If host A and host B are in different subnet, for example, one in NUS, the other is in MIT campus. The routing will be composed of 3 parts: from NUS to local optical network connection port; from Singapore to U.S. in optical network; from U.S. ISP to MIT. So in our implementation, we use ILP separately in 2 subnets on 2 processors, and find a pair of lightpaths using shortest path algorithm.

- To find lightpath, the processors should exchange some information about topology and residual bandwidth.

- After processing a request, the processor who computes the routing should update the residual bandwidth of the links the request used.

In this network scenario, each processor computes routing at the same time. In best case, the processors can process 16 requests distributedly at the same time.

The mathematic formula used in primary/backup path calculation: [1]

(1) On each processor, for each processor, we should apply ILP:

Aij : set of active paths that use link (i,j)

Bij:  set of backup paths that use link (i,j)

Fij:  total amount of bandwidth reserved by active paths that use link (i,j)

Gij: represent the total amount of bandwidth reserved by backup paths that use link (i,j)

Cij: capacity of link (i,j)

Rij: Rij=Cij – Fij – Gij

For each link (i,j), it knows Fij, Gij, Rij, Aij, Bij.

θijuv: the cost (bandwidth usage) of using link (u,v) on the backup path if link (i,j) is used in the active path.

φijuv: Aij  Buv. This is the set of demands that use link (i,j) on the active path and link (u,v) on the backup path.

δijuv= k∈φijuvbk

θijuv=                 0        	 if  δijuv+b  Guv and (i,j) (u,v)

If δijuv+b >Guv and Ruv δijuv+b-Guv and (i,j) (u,v)

δijuv+b−Guv

otherwise

∞

The routing for a request can be formulated as the following integer programming problem:

Xij is set to 1 if link (i,j) is used in the active path.

Yij is set to 1 if link (i,j) is used on the backup path.

Min b (i,j)∈EXij+ (i,j)∈EZij

jXij - jXji = 0      i s,d

jXsj - jXjs = 1

jXdj - jXjd = -1

jYij - jYji = 0        i s,d

jYsj - jYjs = 1

jYdj - jYjd = -1

Zuv   θijuv(Xij + Yij –1)        ∀(i,j) ∀(u,v)

Xij, Yij  ∈ {0,1}

Zij  0

(2) In optical network, due to the wavelength continuity constraint, the known ILP algorithm in optical network protection routing is NP-hard [2]. One possible alternative is to compute lightpaths using shortest path algorithm, i.e. Dijkstra’s algorithm, Bellman-Ford algorithm. And assign wavelength using First-Fit, the first available wavelength. If we have time, we can implement more optical algorithm.

Like the homework 3, part III, we use parallel programs to simulate network. We should use multiple processors to simulate the optical network routing. There are much information exchanges regarding the topology information and bandwidth, wavelength information. We plan to use MPI to implement this part. We expect that the information synchronization will be a big issue.

# Our progress

- Install and become familiar with IBM Optimization Solutions and Library, especially the API functions of LP problem.

- Define the data structure and initialize the network.

- Implementing the sequential routing algorithm using ILP

# * The problems we found so far:

- We tried to find the parallel ILP solver, but the known good ILP solvers (such as CPLEX & AMPL) are commercialized. We have to transfer to IBM’s LP solver, which is free and based on PVM. The problem is that the data file format (.mps) for IBM’s LP solver is very complicated; we still lack a good method to transform the data we know to the data file that the LP solver will recognize. Due to this reason, we are probably unable to make use of parallel ILP solver.

- Our algorithm don’t involve matrix computation, so our implementation is more like a network simulation, not a pure mathematics problem solving. Will this topic be proper for the parallel computing project? Your advices will be greatly appreciated.

# Reference:

[1] Murali Kodialam, T.V. Lakshman, “Dynamic Routing of Bandwidth Guaranteed Tunnels with Restoration”. IEEE INFOCOM 2000

[2] Hui Zang, Jason P. Jue, Biswanath Mukherjeet, “A Review of Routing and Wavelength Assignment Approaches for Wavelength-Routed Optical WDM Networks.”