### Parallel Computing Project Proposal

### Simulation of a Parallel Computing Routing Algorithm

### for IP/DWDM netwrok

### Submitted to Computer Science of SMA

By         Ma Keng Teck (smap1079)

Yu Jia               (smap1062)

Ou Hanyan       (smap1050)

13 March, 2002

Content

Introduction	3

Literature Review	3

Procedure	4

Bibliography	4

## Introduction

For a large-scale network, there should be many routing requests at the same time. Our project is to do the simulation of a parallel computing routing algorithm in IP/DWDM network to solve the requests effectively. We plan to use MPI to implement parallel computing and choose integer linear program (ILP) as well as shortest path algorithms (i.e. Dijkstra’s algorithm) as the routing algorithm to be implemented in parallel. We can use the simulation to evaluate the performance of networks.

## Literature Review

Though IP/DWDM technique has drawn much attention since it appears. IP/DWDM technique is a new technique compared to other network techniques. So, the routing algorithm for DWDM network has just been developed for few years. Now the typical routing algorithm in DWDM network is using centralized integer linear program (ILP) and shortest path algorithm. Since the ILP is NP-hard problem, so people usually use heuristic functions to solve the ILP problem. If you want to know the typical routing algorithm for DWDM, then the paper, A Review of Routing and Wavelength Approaches for Wavelength-Routed Optical WDM networks, by Hui Zang, Jason P. Jue and Biswanath Mukherjee, is a good reference. However, almost all of the existing routing algorithm for DWDM is sequential. Parallel routing algorithm for DWDM network is few. But for the large-scale networks, there should be many routing requests at the same time. Replying to the requests sequentially will take much time. If we can use parallel computing routing algorithm to solve the requests, significant improvement should be achieved.

MPI, the Message Passing Interface, is a standardized and portable message-passing system designed by a group of researchers from academic and industry to function on a wide variety of parallel computers. The standard defines the syntax and semantics of a core of library routines useful to a wide range of users writing portable message-passing programs in C or Fortran 77. A gentle introduction to MPI is: Using MPI: Portable Parallel Programming with the Message passing Interface, by William Gropp, Ewing Lusk and Anthony Skjellum. A complete reference of MPI is: MPI The Complete Reference, by Marc Snir, Steve W. Otto and etc. The parallel application developer will want to have copies of both books handy.

To do routing algorithm for IP/DWDM in parallel, we need master the knowledge of parallel computing and routing algorithm for IP/DWDM. We plan to use MPI to implement parallel computing and choose integer linear program (ILP) based on shortest path algorithm as the routing algorithm to be implemented in parallel.

## Procedure

Our group has three persons and we have about two months to carry out our project.

We divide the whole task, implement the simulation of a parallel computing routing algorithm for IP/DWDM network, into three sub-tasks:

- Developing the central routing algorithm

We choose integer linear program (ILP) based on shortest path algorithm as the routing algorithm to be implemented.

- Developing the simulating engine

- Developing the GUI(with features specially tailored for parallel computing).

Everyone of our group will take a sub-task.

We need personal computers, which we already have in SMA room, and a Linux/Unix cluster consisting of more than ten nodes to run simulating in parallel. Now we can telnet to hydra.aces.nus.edu.sg, which is the master machine of a Linux cluster consisting of 35 nodes of P3 1GHz, and 1 G RAM per node.

We need about one month to accomplish the subtasks individually, half a month to combine the sub-tasks and half a month to test and analysis of performance of our parallel simulation.

## Bibliography

[1] Thomas H. Cormen, Charles E. Leiserson, Ronald L. Rivest, Clifford Stein, Introduction to Algorithms(second edition), McGraw-Hill and the MIT press, 2001

[2] Hui Zang, Jason P. Jue and Biswanath Mukherjee, A Review of Routing and Wavelength Approaches for Wavelength-Routed Optical WDM networks,1999

[3] William Gropp, Ewing Lusk and Anthony Skjellum, Using MPI: Portable Parallel Programming with the Message passing Interface, Cambridge, Mass. : MIT Press, 1999.

[4] Marc Snir, Steve W. Otto, Steven Huss-Lederman, David W. Walker, Jack Dongarra, MPI The Complete Reference, Cambridge, Mass. : MIT Press, 1998.