Simulation of a Parallel Routing Algorithm for IP/DWDM network

SMA 5505 Applied Parallel Computing

Moy Group

Ma Keng Tech (smap1079)

Ou Hanyan      (smap1050)

Yu Jia               (smap1062)

May 20, 2002

# Description of work

In this project, we designed and implemented a parallel routing algorithm, which dynamically allocates network bandwidth resources for IP and Optical Networks.

This report is divided into 5 parts:

Part 0: Introduction

Part 1: Methodology

Part 2: Implementations

Part 3: Results and Discussion

Part 4: Future Work

Platform information:

Programming language: C, JAVA

Platform: LINUX  -- Beowulf.lcs.mit.edu

UNIX    -- Mpp.comp.nus.edu.sg

Member contributions:

Yu Jia: IP/DWDM Network Design, Optical Network Routing Algorithm, Request Generation

Ou Hanyan: IP Network Routing with Integer Linear Programming (ILP) Solver, IP Network Topology Generation

Ma Keng Teck: Parallel Wavelength Reservation Protocol (SWAP Protocol), Graphic User Interface, and Debugging.

# Part 0: Introduction

## 1. Motivation

In recent years, advances in dense wavelength-division multiplexing (DWDM) technology [1] have enabled the deployment of systems that are capable of providing large amounts of bandwidth (10Tb/s/optical fiber). It is widely expected that all-optical connections can be established between ingress router and egress router, with the introduction of optical crossconnects (OXCs).

Considering compatibility with traditional IP network and economic feasibility, optical network is only applied as core network. The hybrid network structure will remain for a period of time. In view of this, the Internet Engineering Task force (IETF) is currently working on generalized multiprotocol label switching (GMPLS)[2], a generalized control framework for establishing various types of connections, including packet switching, time division multiplexing (TDM), wavelength switching and space switching etc.

The hybrid network structure (traditional IP network, new DWDM optical network) and various types of connections bring new challenges to today’s service providers. Competition is fierce, margins are elusive, and the cost of innovation is high. To help service providers and network equipment manufacture to design resilient, cost-effective optical network, we design and implement a simulation of a parallel routing algorithm for IP/DWDM network infrastructure.

## 2. Background

An optical network allows dynamic provisioning of optical layer connections, called lightpaths. A lightpath is defined here as an all-optical channel, which may span multiple fiber links to connect a source edge router with a destination edge router. Due to the high cost of wavelength conversion devices, we assume lightpath occupies the same wavelength on each link in its route, a restriction known as the wavelength-continuity constraint.

Thus, given a set of lightpaths that need to be established, and given a constraint on the number of wavelengths, it is necessary to determine both the routes over which these lightpaths should be established and the wavelengths that should be assigned to these lightpaths. This problem is known as the routing and wavelength assignment (RWA) problem[3].

In a wavelength-routed network, the traffic can be either static or dynamic [4]:

- In a static traffic pattern, a set of lightpaths are set up all at once and remain in the network for a long period of time. The RWA problem for static traffic is known as the static lightpath establishment (SLE) problem.

- In a dynamic traffic pattern, a lightpath is set up for each connection request as it arrives, and the lightpath is released after some finite amount of time. The problem of lightpath establishment in a network with dynamic traffic demands is called the dynamic lightpath establishment (DLE) problem [4]. DLE is our focus in this project.

### Dynamic Routing and Wavelength Assignment

When lightpaths are established and taken down dynamically, routing and wavelength assignment decisions must be made as connection requests arrive at the network. It is possible that, for a given connection request, there may be insufficient network resources to set up a lightpath, in which case the connection request will be blocked. The connection may also be blocked if there is no common wavelength available on all of the links along the chosen route. Thus, the objective in the dynamic situation is to choose a route and a wavelength that maximize the probability of setting up a given connection, while at the same time attempt to minimize blocking for future connections.

Approaches to solving the routing subproblem can be categorized as either static or adaptive and as utilizing either global or local network state information. In this project, we applied Adaptive Routing Based on Global Information and Parallel Wavelength Assignment.

- Adaptive Routing Based on Global Information

With the assumption that the network topology usually remains static, each router maintains the network topology information, and is responsible for finding routes and setting up paths for coming connection requests.

- Parallel Wavelength Assignment

Based on the global topology information, a node can calculate an optimal route to a destination on a given wavelength. The source node then attempts to reserve the desired wavelength on each link in the route by sending a separate control message to each node in the route. Each node that receives a reservation request message will attempt to reserve the specified wavelength, and will send either a positive or negative acknowledgment back to the source. If the source node receives positive acknowledgments from all of the nodes, it can use the lightpath and begin communicating with the destination.  The advantage of a parallel reservation scheme is that it shortens the lightpath establishment time by having nodes process reservation requests in parallel. What’s more, it enables the real dynamic lightpath establishment.

# Part 1: Methodology

## 1. Our approach

There are some products in the market, which can simulate and evaluate optical network traffic. One of them is WDM Guru, produced by OPNET Technologies, Inc. (http://www.opnet.com/products/wdmguru/home.html). Compared with these products, our approach is unique, we

- Use parallel machine to simulate the routing behavior of about 70,000 routers. (This limit is actually given by the machine memory we have)

- Support LINUX and UNIX, while other products support Windows NT/2000.

- Compute routes covering multiple IP networks and Optical network, while other products simulate Optical network only.

- Use friendly Graphical User Interface (windows-based).

- Considers communication cost for wavelength reservation, while sequential versions don’t.

## 2. Network Design

Since the NSF topology (used in U.S.) we get composed of 14 OXCs, we let each processor on a cluster machine simulate a border router that is connected to an OXC. Each border router is connected to an IP network. Therefore, totally 14 IP networks are connected to the OXCs in the optical network.

IP Network

We have simulated maximum 5000 routers for each IP network on each processor.  The connection request may cover multiple networks. A typical request is IP network  Optical network  IP network.

IP Network

IP Network

Figure 1 An example of 8 IP networks connected to the core Optical network

IP Network

IP Network

IP Network

IP Network

IP Network

Optical Core

## 3. Algorithm

- For each request from host A to host B,

a.  If host A and host B are in the same IP network, we use ILP (Integer Linear Programming) solver to find an optical path.

b.   If host A and host B are in different subnet, for example, one in NUS, the other is in MIT campus. The routing will be composed of 3 parts: from NUS to local optical network connection port; from Singapore to U.S. in optical network; from U.S. ISP to MIT. So in our implementation, we use ILP separately in 2 IP subnets on 2 processors, and find a lightpath using shortest path algorithm.

c. The wavelength availability information is stored distributedly. Therefore, after a shortest path is found in optical network, the source OXC will inform other OXCs (use MPI functions) on the route for the wavelength reservation. If the source OXC receives positive acknowledgement on a specific wavelength λ, it knows the lightpath is setup. Otherwise, the source OXC will recompute another route and reserve wavelength on the new route. For wavelength reservation, we applied SWAP protocol. (It will be described in the Implementation part.)

d. After processing the request, the border router will store the path information to local table for later usage.

# Part 2: IMPlementation

## 1. Network Topology and Requests Generation

1) Network Topology Generation

To simulate network routing problem, the first thing is to generate network topology for each IP network on each processor.

The requirements for a topology include:

(1). Each edge of a topology is bi-directional. This means if a host in a topology, named A, can reach another client of a topology, named B, then B can also reach A.

(2). The topology should be a connected graph. That means every host can reach any other hosts in the local network.

(3) The out degree of each node should be at least 2. Otherwise, in case of link failure, the host will be isolated.

Based on these requirements, and to simulate the network topology in reality, we generate random network topology on each processor. Below is a small example of a topology (clients number is 10):

Figure: An Example of an IP Network Topology

Of course, it’s easy to modify the code to make the program read into the fixed network topology if necessary.

2) Requests Generation

To simulate network routing requests, we generate requests randomly and the requests include:

(1). Local request: This kind of requests’ source and destination are in one IP subnet. Local communication generates this kind of requests. We just need to route in an IP subnet to solve the request problem. One local request = one IP request.

(2). Global request: The source and

destination of this kind of requests are in two IP subnets. Global communication generates this kind of requests. We need to route over two IP subnets and optical network to solve the requests problem. One global  = two IP requests + one Optical request. Below is an example of these two kinds of requests.

100                                        200

58

82

285

234

OXC

### OXC

2

IP subnet                                                                               IP subnet

Figure: Example of the local requests and global requests.

Local requests:  request 58-82, request 285-234;

Global requests: request 82-285 (this request can be considered as three part: local request 82-100, optical request 100-200, local request 200-285)

## 2. Parallel Computing Routing Algorithm for IP/DWDM network

Now we have network topology and routing requests, we need to solve them. We think out a parallel routing algorithm for IP/DWDM network. Because there are two kinds of network in IP/DWDM network: IP network and optical network, we do parallel computing routing in these two networks.

2.1 Routing in IP network in parallel

To have an overview of routing in IP network, first let’s see what’s the sequential routing in IP network and what’s the parallel routing in IP network.

The algorithm for routing in IP network in sequence will deal with the requests one by one. Suppose there are several global requests to route. Every request needs to route in IP subnet 1 first, then route in optical network, finally route in IP subnet 2. Request 1 will be routed first, then request 2, request 3 and so on. If we use different line type to represent different time order, we can represent sequential routing like the following figure:

1 2   3

1   2   3

IP subnet 1                                               IP subnet 2

Figure:  the sequential routing algorithm

represent being dealt with in first order

represent being dealt with in second order

represent being dealt with in third order

The algorithm for routing in IP network in parallel will deal with the global requests as the local requests in two IP subnets and calculate the two IP subnets’ local requests in parallel. Then calculate the request for optical network. If we use different line type to represent different time order, we can represent parallel routing like the following figure:

1 2   3

1   2   3

IP subnet 1                                               IP subnet 2

Figure:  the parallel routing algorithm

represent being dealt with in first order

represent being dealt with in second order

2.2 How to route the IP requests in parallel

We think out such an algorithm to route the IP requests in parallel (the IP requests are generated randomly on each processor):

- Classify the requests into local requests and global requests.

- Each processor sends the global requests destination to according processors.

- Each processor receives the global requests destination from other processors and translates the received destination to local IP.

3.   Each node calculates the routing path for local requests in parallel. I

- P Network Routing with ILP solver.

We can elaborate the algorithm by showing an example.

Each processor manages an IP subnet. Suppose there are 100 hosts in each subnet. The first client, whose local IP address is 0, is the border router (connected directly with OXC).

Subnet 1(processor 0) has the clients whose global IP address are from 0 to 99. Client 0 is the OXC. Subnet 2(processor 1) has the clients whose global IP are from 100 to 199, but in fact, the clients local IP in subnet 2 are from 0 to 99. Client 100, or say, local IP 0 client, is the OXC.

Subnet 1 has 30 global requests whose destination are among 100~199 besides local requests. Subnet 2 has 53 global requests whose destination are among 0~99 besides local requests.

In our algorithm, each subnet looks through all its requests first. If the request is a local request, don’t change it and pass it to the local request queue. If the request is a global request, change the destination to 0 and pass it to the local request queue. At the same time, create a request whose source is 0 and the destination is the original global request’s destination and pass it to the according subnet’s global request queue. So, in the above example, subnet 1 has 30 global requests in the request queue of subnet 2. Subnet 2 has 53 global requests in the request queue of subnet 1.

After classification, each processor sends the global request queue to the according processor and receives the global request queue from other processors. In the above example, processor 0 sends a request queue of length 30 to processor 1 and receives a request queue of length 53 from processor 1. At the same time, processor 1 sends a request queue of length 53 to processor 0 and receives a request queue of length 30 from processor 0.

The third step is to change the received requests to local requests by changing the destination to local IP. Then add the received requests to local request queue.

Now all the requests are local requests and each processor routs its local requests independently.

2.3 Routing in IP network using ILP [5]

Now we know how to route the IP requests in parallel. The left problems are what method to use to route the requests in each subnet and how to make the method run in parallel on each processor.

We choose ILP as the method to use to route the requests in each subnet. There are two reasons for using ILP:

- We want to get the optimal paths.

- ILP is more flexible than other algorithm.

If we want to add some constraints or we want to change the object, it’s very easy to do in ILP: just change the ILP formulas. But for some other algorithm, the entire algorithm needs to change if some constraints change or the object changes.

Find one optimal path for routing requests can be formulated as the following ILP problem (this is what we implemented in the routing algorithm):

Xij is set to 1 if link (i,j) is used in the active path.

Yij is set to 1 if link (i,j) is used on the backup path.

Object formula:

Min b (i,j)∈EXij

Constraints formula:

jXij - jXji = 0      i s,d

jXsj - jXjs = 1

jXdj - jXjd = -1

Xij  ∈ {0,1}

If we want to change the object and constraints to find one primary path and one backup path, the requests can be formulated as the following ILP problem:

Aij : set of active paths that use link (i,j)

Bij:  set of backup paths that use link (i,j)

Fij:  total amount of bandwidth reserved by active paths that use link (i,j)

Gij: represent the total amount of bandwidth reserved by backup paths that use link (i,j)

Cij: capacity of link (i,j)

Rij: Rij=Cij – Fij – Gij

For each link (i,j), it knows Fij, Gij, Rij, Aij, Bij.

θijuv: the cost (bandwidth usage) of using link (u,v) on the backup path if link (i,j) is used in the active path.

φijuv: Aij  Buv. This is the set of demands that use link (i,j) on the active path and link (u,v) on the backup path.

δijuv= k∈φijuvbk

θijuv=                 0        	 if  δijuv+b  Guv and (i,j) (u,v)

If δijuv+b >Guv and Ruv δijuv+b-Guv and (i,j) (u,v)

δijuv+b−Guv

otherwise

∞

Min b (i,j)∈EXij+ (i,j)∈EZij

jXij - jXji = 0      i s,d

jXsj - jXjs = 1

jXdj - jXjd = -1

jYij - jYji = 0        i s,d

jYsj - jYjs = 1

jYdj - jYjd = -1

Zuv   θijuv(Xij + Yij –1)        ∀(i,j) ∀(u,v)

Xij, Yij  ∈ {0,1}

Zij  0

2.4 Make ILP solver run in parallel

Now we have chosen ILP as the method to use to solve routing problems, but we must make the ILP solver run in parallel on each processor.

At first, we want to use parallel ILP solver. But there is no free parallel ILP solver. I downloaded a free sequential ILP solver (its author: Michel Berkelaar). But this ILP solver can’t run on several processors in parallel.

We have to make the sequential ILP solver run in parallel on each processor ourselves. There are two methods we think out to make the ILP solver run in parallel.

One method is to write a shell script like:

#!/bin/sh

ssh compute-0-0 ./lp_solve < ./test0.lp

ssh compute-0-1 ./lp_solve < ./test1.lp

ssh compute-0-2 ./lp_solve < ./test2.lp

...

But a shell script can’t be involved as a part of a program. And we need to keep generating requests and solving requests over and over. That means solving requests should be a part of an iteration. A shell script can’t play such a role.

So we try another method: modifying the source code of the ILP solver to make the ILP solver as a module of the whole program. I write a function to replace the original main function of the ILP solver. Then call the function in our main function to solve the requests. Fortunately, it really works! So, we can put the solver in the main iteration now. We use mpirun to make each processor keep running the iteration in parallel.

So, now we have solved the two problems we mentioned in the beginning of section (2.3).  Based on section (2.2) and section (2.3), we can route in IP network in parallel successfully.

2.5 Network Resource Management

To satisfy each request in IP networks and optical network, we should assure the route computed has enough bandwidth for the data transmission. When the requests are outdated, we should release the bandwidth resources.

2.5.1 Before Requests Come In

The application should initialize the networks according to the topology generated. The involved data structures are: router, OXC, Link, Fiber (Optical).

Before computing a request, the program should scan through the data to remove the links that don’t have enough bandwidth or wavelength. It also needs to assign the costs to each links. For example, for good-quality and short links, we assign small cost values. For low quality and long-distance links, we assign large cost values. When we compute the shortest path, the goal is minimized cost, so the algorithm will choose the good-quality links as much as possible.

2.5.2 After computing each request

In IP network, we will update the residual bandwidth information on the route computed. If no enough bandwidth, a warning will be given.

In Optical network, we will store the lightpath newly setup to the routing table in source OXC. Because it can save recomputation for next requests who has same source and destination OXCs.

2.5.3 Release Resources

We use Lazy Release of resources: put the active requests to queues, which use same ligthpath. Whenever a new request comes in and request using this lightpath, OXC will check the timestamps of the requests stored in the queue. If they are outdated, delete them and increase the residual bandwidth on the lightpaths.

If a lightpath hasn’t been used for a period of time, the algorithm will release this lightpath and inform other OXCs in the optical network, the wavelength this lightpath used is available now.

2.6 Routing in Optical Network

In optical network, due to the wavelength continuity constraint, the known ILP algorithm in optical network protection routing is NP-hard. One possible alternative is to compute lightpaths using shortest path algorithm, i.e. Dijkstra’s algorithm, Bellman-Ford algorithm[6].

Borrowed the idea from OSPF (Open Shortest Path First) protocol, which is popular in the real cases, we applied Dijkstra’s algorithm.

An interesting thing of Dijkstra’s algorithm is that we also use it to check the connectivity of a graph. After a random request generation, we can use Dijkstra’s algorithm to compute the shortest path starting from a source node. If any node has large (Maximum) distance with the source node, we know the topology is disconnected. We need to adjust the topology.

2.7 Simple Wavelength Assignment Protocol (SWAP[7])

This is the layer where the individual nodes of the network will need to send exchange control messages to setup the wavelength route. As our focus is on DLE problem, we would need to simulate some protocol fo optical switching signaling. After much research, we have decided to implement SWAP based on the following reasons:

- Simple

  - To implement and understand.

This is crucial to this project, as due to the limited resources and the extensive scope, we could not afford to spend too much resources on any particular layer of the simulation

- Algorithm is open.

  - Unlike some commercial protocols, the algorithm of this protocol is available for us to study and implement.

The details of SWAP can be found in [7]. We have only implemented the “first-hop-initiated SETUP and teardown (RELEASE) in this version of the project. This is because, this type of assignment is common and that it integrates well into our framework. Other possible setup and teardown types include last-hop-inittiated.

Briefly, SWAP works in 2 phases to reserve a wavelength path, and release wavelength in 1 phase. Each phase takes a full route from the source to the destination.

SETUP Protocol

Phase 1: Setup

- Source reserved its free set. Ask the next hop to reserve wavelengths by sending it the set of wavelengths of free wavelengths.

- Next hop will intercept the received set and its own free set of wavelengths. This new set is then reserved. And the reserved set is sent to the next hop in the route.

  - If new set is empty, send an error message to previous hop, error message includes the free and used set of this node. This will enable the source router to recompute a new route if necessary.

- Continues 2 until last hop is reached.

- Last hop will choose a wavelength from the received set and its own free set of wavelengths. In our implementation, the first available wavelength is chosen. Then it goes into phase 2.

Phase 2: COMMIT

- Last hop sends the intended wavelength to be used to previous hop.

- Previous hop releases the reserved wavelengths to the free set. And moves the intended wavelength from the free set to the used set. Relays the message to its previous hop.

- Repeats 2 until first hop is reached.

- First hop releases the reserved wavelengths to the free set. And moves the intended wavelengths from the free set to the used set. Now this wavelength can be used to send data.

RELEASE Protocol

- Source moves the used wavelength to the free set. Source will send a release message to the next hop in the route.

- Last hop and intermediate hops will move the used wavelength to the free set. Then relay the message to the next hop if any.

Implementation

- This portion of the program is coded in MPI.

- Due the possibility of deadlock, MPI_Isend is called instead of MPI_Send. Also, MPI_Iprobe is called to check if there is a message to be received, if so then MPI_Recv is invoked.

- The functions are designed in an Abstract Data Type (ADT) manners. There are some global variables which are not visible to the other part of the programs. Hence, the protocol functions can be used like a library and can be plugged into other programs in the future.

The main program functions in a loop, each time probing the network for incoming messages. Then the messages are handled accordingly.

After receiving commit message, the pending IP requests are immediately sent to the destination.

When received error message for congested links, the setup is retried until the number of maximum retries specified is exceeded. Then the program will report an error and continues.

When received data message, the computation for local IP routing routine is executed.

When received a terminate message, the loop will terminate.

Some Points to Take Note

- The last hop is not the destination, it is the hop before the destination. This is because the destination has no control over the incoming wavelength. For example, in a 2 hop route, this protocol is not necessary as the source can just send the data over all unused wavelengths.

- There are various other options to consider. For example, reserving a random wavelength for the first hop rather than the whole free set. This alternative design is more suited when the network is lightly loaded, and any chosen wavelength is likely to succeed. Also the last hop can randomly choose a wavelength instead of the first available. This is particularly attractive if the first hop do not reserve its whole free set in the SETUP phase.

Future Work

- To implement the full SWAP as well as other WAP for comparisons of the real network simulations.

- To read configurations and settings from file. This version of program has a lot of hard-codings for its settings. For example, the number of nodes in the network. We would like to extend the code so that user can specify setting files to read in and the program can then stimulate the different network accordingly.

GUI

The user interface was coded in Java. Java SWING API makes coding for windowed system an easy task. Integration of the MPI backend with the Jave frontend is achieved using Unix pipe. However, this means that the UI is one half-duplex, as the Jave frontend cannot control the MPI backend.

The team has ran into various problems due to the non-thread-safe property of the SWING components. As a result, the user-interface is not robust. Also, the responsiveness of the GUI is not ideal, both due to the slowness of Java on the Unix machines as well as the threading problem mentioned.

# Part 3: results and discussion

In this section, we present experimental results that compare the performance of the parallel and sequential routing programs.

Our algorithm is focused on DLE problem, and it is still open for research now, so there should be no standard for routing algorithms in IP/DWDM networks. In addition, we fail to find an existing sequential code, because each research group in the world takes different assumptions and initial data. Considering this fact, we wrote a sequential code by ourselves to compare the performance with the parallel programs.

The performance of our application can be evaluated in two parts, they are:

- In multiple IP networks (without considering the time in Optical network)

- Wavelength Allocation in Optical Network.

1. In Multiple IP Networks

(1) Comparison of the time used for computing 100 requests, using Sequential version and parallel version (2 processors)

#Requests = 100 (run on Beowulf.lcs.mit.edu)

| #Hosts in Each Subnet | Sequential time (sec) | Parallel time (sec) |
| --- | --- | --- |
| 500 | 11 | 7 |
| 1000 | 36 | 17 |
| 1500 | 70 | 30 |
| 2000 | 125 | 53 |
| 2500 | 175 | 90 |
| 3000 | 212 | 116 |
| 3500 | 329 | 138 |
| 4000 | 381 | 212 |

(The program we tested for this benchmark is included in our source code package. See Appendix)

From the above figure, we see in some trials the speedup for 2 processors /sequential version exceeds 2. This is because we applied sequential algorithm using 2 processors. The sequential version communicates two times for each request (one for sending request, one for acknowledgement). While the parallel version only need to communicate once in the front of the computation. The two processors then will compute the requests in batch.

The difference of the sequential and parallel algorithm is described in section 2.2.1 (on page 8).

Similarly, we tested the parallel versions with 4 and 8 processors.

# total requests = 100,

| # clients in a subnet | Sequential time (sec) | Parallel time, 4 procs (sec) | Parallel time, 8 procs (sec) |
| --- | --- | --- | --- |
| 500 | 18 | 6 | 3 |
| 1000 | 55 | 13 | 6 |
| 1500 | 79 | 20 | 12 |
| 2000 | 132 | 35 | 17 |

2. Wavelength Allocation in Optical Network

We feel part 2 is difficult to be evaluated for speedup, because we use 14 processors to simulate a distributed network scenario. While the sequential version doesn’t consider the communication cost at all, and stores all the wavelength information in a central place (e.g. use global variable). If we only take time into account, the distributed version (our parallel program) will be slower than the sequential version.

The central information storage used in sequential version is not suitable for real network for two reasons. Firstly, it’s hard for central resource management to control the freshness of the information, because the wavelength availability information may be outdated quickly. To keep the freshness, the central resource management part should be informed of all the wavelength usages. It will bring the bottleneck to the system. Secondly, the central place is prone to the “single point of failure” problem.

Therefore, the best thing to test part 2 is to run our program and see the performance.

# Part 4: future work

The requests in our program are just randomly generated, and we do not consider the distribution of customer’s network usage in real case. To make the simulation more realistic, we should consider using replaying history data (traced-based) or some statistics functions.

We have considered and rejected using Java Native Interface (JNI) to integrate the GUI with the MPI backend. For JNI to works, we would either run java and calls the C routines; or run the Java Virtual Machine and the MPI at the same time.

For the first approach, it is relatively simple to call C routines in Java, however, we cannot run the MPI on multiple nodes at the same time. For the second method, the user will have to invoke the JVM, when running the program. And it is quite difficult to implement.

Some elegant approaches would be to code the UI in C or to use TCP/IP.

Acknowledgement

We would like to thank Prof. Hsu Wen Jing, Prof. Alan Edelman, Mr. Wang Hao, and Mr. Roy Choy for their kind suggestions to our project.

REFERENCE

[1] Technologies and Architectures for Scalable Dynamic Dense WDM Networks, Jaafar M.H. Elmirghani, Hussein T. Mouftah, IEEE Communication Magazine, February 2000

[2] A Simple Dynamic Integrated Provisioning/Protection Scheme in IP over WDM Networks, Yinghua Ye, Chadi Assi, Sudhir Dixit, Moharned A. Ali, IEEE Communications Magazine, November 2000

[3] A Review of Routing and Wavelength Assignment Approaches for Wavelength-Routed Optical WDM Networks, Hui Zang, Jason P.Jue, and Biswanath Mukherjee, University of California, Davis

[4] Dynamic Lightpath Establishment in Wavelength-Routed WDM Networks, Hui Zang, Jason P.Jue, Laxman Sahasrabuddhe, Ramu Ramamurthy, Biswanath Mukherjee, IEEE Communications, September 2001.

[5]Dynamic Routing of Bandwidth Guaranteed Tunnels with Restoration, Murali Kodialam, T.V.Lakshman, IEEE INFOCOM 2000

[6] Introduction to Algorithms, Thomas H. Cormen, Charles E.Leiserson, Ronald L. Rivest, Clifford Stein, The MIT Press, Cambridge, Massachusetts.

[7] Simple Wavelength Assignment Protocol, Stephen Suryaputra, Nortel Networks, Joseph D. torch, Joseph Bennister USC/ Information Sciences Institute

Appendix

Output

For GUI, output is

S D N

S : Source node

D: Destination node (Source immediate neighbour)

N: Number of used wavelengths

There are multiple *.LP, *.LOG files in the current directory. The *.LP files are the generated input files for ILP Solver. The *.LOG files record the routing information in each IP Subnets.

For more detailed output, please uncomment the

#define VERBOSE

#define DEBUG

In the config.h file, the various settings of the network can be modified to test the result.

/* Constants */

#define GRP_SIZE 14			/* Number of nodes in the network */

#define MAX_WAVELENGTH 4	/* Number of total wavelengths in a link */

#define UNIQUE_ID 1000		/* Maximum number of unique ids */

#define TICK 0.0001			/* Time wait before generating next set of IP requests, in seconds */

#define TTL 0.001			/* Time to Live before a route is being released since last used */

#define MAX_RETRIES 10		/* Number of retries for a link before giving up */

#define MAX_REQUESTS 10	/* The number of IP requests to generate in the simulation before exiting */

#define ALL_ONES ((1 << MAX_WAVELENGTH) - 1) /* the bit pattern for all ones */

#define largeN 100   /*largeN*largeN matrix: Number of IP hosts in subnet */

#define degree 3.0   /* Aevrage outgoing links for each IP node */

#define limit  (double)degree/(double)largeN /* Function of degree */

#define SNUMBER  10 /* Number of requests generated */

## How to run and see the result for the router program

In our source code, there is a zip file named router.tar.gz, which is the source code of our router program. To run it to see how the router works, you should follow the following steps: (Note:  You must use AP3000 Distributed Memory Parallel Processing System of SoC because we run the program and write the make file based on this cluster. And this cluster’s command is not mpicc or mpirun. So, you must use this cluster and follow the following steps.)

1). telnet mpp.comp.nus.edu.sg, it appear: login:, use your account on sunfire to login in.

2). Put router.tar.gz in a folder of sunfire. (mpp use the same disk space with sunfire)

3). gunzip router.tar.gz

4). tar xvf router.tar, then you get a folder named router.

5). cd ./router

6). more myrun, you will see:

chmod 755 mycc

./mycc gensolvempi.c gensolvempi llist.c SWAP.c liblpk.a OptNetwork.c -ll

aprun -nproc 14 gensolvempi

7). Type these three commands one by one, then you’ll see:

ouhanyan@cnet0029:~/router[514]$ chmod 755 mycc

ouhanyan@cnet0029:~/router[515]$ ./mycc gensolvempi.c gensolvempi llist.c SWAP.

c liblpk.a OptNetwork.c -ll

gensolvempi.c:

llist.c:

SWAP.c:

OptNetwork.c:

Linking:

ouhanyan@cnet0029:~/router[516]$ aprun -nproc 14 gensolvempi

11 12 1

7 8 1

0 7 1

6 9 1

11 12 1

0 1 1

8 9 1

7 8 0

2 1 1

4 2 1

0 7 1

4 0 1

5 4 1

4 1 1

6 3 1

6 9 1

8 11 1

7 0 1

Terminated!

That’s the result of our program. It terminates after finding the above light-paths because we use #define MAX_REQUESTS 10 to set the router terminate after solving the number of requests. You can reset the argument to make the program run longer, like 50, 100 or so on.

The light-paths routing result for optical network appears on the screen, like the above show, while the routing result for IP network is stored in according log file. For example, rank0 IP subnet’s routing result is recorded in the file rankA.log,rank1 IP subnet’s routing result is recorded in rankB.log and so on until rankN.log, which is for rank13. There are at most 14 subnets in our simulation.

You can use command: more rankA.log to see the routing result for rank0 IP subnet, like following:

ouhanyan@cnet0029:~/router[519]$ more rankA.log

rank 0, request 0, sour = 80, dest = 76.

solution of rank 0 for S=80, D=76:

80      33      71      20      69      77      86      76     -1

rank 0, request 1, sour = 63, dest = 82.

solution of rank 0 for S=63, D=82:

63      1       29      40      82     -1

rank 0, request 2, sour = 9, dest = 10.

solution of rank 0 for S=9, D=10:

9       54      22      95      10     -1

rank 0, request 3, sour = 66, dest = 59.

solution of rank 0 for S=66, D=59:

66      1       63      61      43      0       59     -1

rank 0, request 4, sour = 33, dest = 59.

solution of rank 0 for S=33, D=59:

33      71      28      18      64      43      0       59     -1

rank 0, request 5, sour = 1, dest = 67.

solution of rank 0 for S=1, D=67:

1       66      7       67     -1

rank 0, request 6, sour = 69, dest = 63.

solution of rank 0 for S=69, D=63:

69      99      68      37      63     -1

rank 0, request 7, sour = 21, dest = 17.

solution of rank 0 for S=21, D=17:

--More--(8%)

## How to run and see the result for the router program

The source code for testing speedup is packaged in testIP.tar.gz

1) Please type following commands to unzip the files:

Gunzip testIP.tar.gz

Tar –xvf testIP.tar

Cd testIP/testIP/Iprouter/

2) Please use following commands to compile and run the sequential version (must use 2 processors). The time is recorded in rank0.log.

mpicc -O   -c -o gensolveseq.o gensolveseq.c

mpicc -o gensolveseq -O gensolveseq.o liblpk.a -ll -lm

mpirun -np 2 n1-8 gensolveseq >resultseq

more rank0.log

3) Please use following commands to compile and run the parallel version (use –np 4 for 4 processors and –np 8 for 8 processors). The time is recorded in rank0.log

mpicc -O   -c -o gensolvepal.o gensolvepal.c

mpicc -o gensolvepal -O gensolvepal.o liblpk.a -ll -lm

mpirun -np 4 n1-8  gensolvepal >result

more rank0.log

Note:

To change number of IP hosts and requests, please change the defined value in gensolve.h, for example

#define LargeN 1000 //the subnet is composed of 1000 IP hosts

#define SNUMBER 100 //the requests generated.

To test 100 request, we suggest you use SNUMBER=100 for sequential code, SNUMBER=50 for parallel code run on 4 processors, SNUMBER=25 for parallel code run on 8 processors. Because with this adjustment, we can assure the total computed requests are 100.