Parallel Sparse Incomplete Cholesky Preconditioners

Group Members

Aditya Kumar Gupta

Wu Yong Hua

Low Wai Chong

Contents

1.	Background	1

2.	Incomplete factorization preconditioners	1

3.	Incomplete Cholesky preconditioner	1

4.	Graph Theory Definitions	2

5.	Parallelism in sparse factorization	2

6.	Parallel Algorithm	4

7.	Storage Schema	5

8.	Level of Fill	6

9.	Reordering	7

10.	CutHill-McKee/Reverse CutHill-McKee Reordering	8

11.	Peripheral Nodes	9

12.	Matrix Diagrams	10

13.	Test Results	13

14.	Future Work	18

Acknowledgements	18

Appendix A: Running the submitted programs	19

Appendix B: Work Division	20

References	20

Parallel Sparse Incomplete Cholesky Preconditioners

Aditya Kumar Gupta, Low Wai Chong, Wu Yonghua

{smap1067@nus.edu.sg, smap1078@nus.edu.sg, smap1059@nus.edu.sg}

# 1.	Background

A large subset of problems in engineering fall into the realm of solving linear equations of the form Ax = b. One of the methods to solve these equations is by means of iterative methods. This technique uses successive approximations to obtain more accurate solutions to the linear system at each step.

The rate at which an iterative method converges depends greatly on the spectrum of the coefficient matrix A. Hence, iterative methods usually involve a second matrix that transforms the coefficient matrix into one with a more favorable spectrum. The transformation matrix is called a preconditioner. A good preconditioner improves the convergence of the iterative method, sufficiently to overcome the extra cost of constructing and applying the preconditioner.

# 2.	Incomplete factorization preconditioners

A broad class of preconditioners is based on incomplete factorizations of the coefficient matrix. We call a factorization incomplete if during the factorization process certain fill elements, non-zero elements in the factorization in positions where the original matrix had a zero, have been ignored.

# 3.	Incomplete Cholesky preconditioner

Incomplete Cholesky preconditioner performs a incomplete Cholesky factorization on the coefficient matrix A. Cholesky factorization of A is given as

A = LLT

where A is symmetric positive definite and L is lower triangle with positive diagonal entries.

Our algorithm computes the Cholesky factorization of sparse matrices. A matrix is sparse when there are a large number of zero entries in the matrix.

Algorithm 1 gives a standard, column oriented formulation in which the Cholesky factor L overwrites the initial matrix A, and only the lower triangle is accessed:

for j = 1, n for k = 1, j - 1 for i = j, n {cmod(j, k)} aij = aij - aik . ajk ajj = √ajj for k = j + 1, n {cdiv(j)} akj = akj / ajj

Figure 1 Cholesky factorization algorithm

The outer loop in Figure 1 is over successive columns of A. The current column (indexed by j) is modified by a multiple of each prior column (indexed by k); we refer to

such an operation as cmod(j, k). After all its modifications have been completed, column j is then scaled by the reciprocal of the square root of its diagonal element; we refer to this operation as cdiv(j). As usual, this is but one of the 3! ways of ordering the triple nested loop that embodies the factorization.

One interesting condition is that column j of the Cholesky factor L does not depend on prior column k if ljk = 0, which not only provides a computational shortcut,

but also suggests a source of parallelism.

# 4.	Graph Theory Definitions

We use graph theory for the purpose of analyzing the matrix and to identify the parallelism that can be extracted from the matrix. We begin with some definitions

1.	Vertex Elimination

Two ordered, undirected graphs are associated with the sparse, symmetric matrix A. First, (A), the graph of A, is the graph with vertices {1, 2, 3, …. n} and edges

E(A) = {(i, j) | aij ≠ 0} Next G*(A), the filled graph, with vertices {1, 2, 3,…. n} and edges E*(A) = {(i, j) | lij ≠ 0}

2.	Elimination trees

Let A have a cholesky factor L. The elimination tree T(A) is a rooted spanning forest of G*(A) defined as follows. If vertex u has a higher numbered neighbor v, then the parent p(u) of u in T(A) is the smallest such neighbor: otherwise u is a root.

# 5.	Parallelism in sparse factorization

Parallelism can be gained, in the sparse case, at the level of subtrees of the elimination tree. If Ti and Tj are disjoint subtrees of the elimination tree, with neither root node a descendant of the other, then all of the columns corresponding to nodes in Ti can be computed completely independently of the columns corresponding to nodes in Tj, and vice versa, and hence these computations can be done simultaneously by separate processors with no communication between them. For example, each leaf node of the elimination tree corresponds to a column of L that depends on no prior columns, and hence all of the leaf node columns can be completed immediately merely by performing the corresponding cdiv operation on each of them. Furthermore, all such cdiv operations can be performed simultaneously by separate processors (assuming enough processors are available).

We can see that the elimination tree serves to characterize the parallelism that is unique to sparse factorization. In particular, the height of the elimination tree gives a rough measure of the parallel computation time, and the width of the elimination tree gives a rough measure of the degree or multiplicity of large grain parallelism. These measures are only very rough, however, since the medium level parallelism also plays a major role in determining overall performance. Still, we can see that short, bushy elimination trees are more advantageous then tall, slender ones in terms of the large grain parallelism available. And just as the fill in the Cholesky factor is very sensitive to the ordering of the matrix, so is the structure of the elimination tree.

Below are a few examples of the generation of G(A)/G*(A) (this is because in case of fill level of zero G(A) = G*(A)) and T(A).

Figure2 Elimination trees

Figure 2 shows a small one dimensional mesh with a ``natural'' ordering of the nodes, the nonzero patterns of the corresponding tridiagonal matrix A and its Cholesky factor L, and the resulting elimination tree T (A). On the positive side, the Cholesky factor suffers no fill at all and the total work required for the factorization is minimal. However, we see that the elimination tree is simply a chain, and therefore there is no large grain parallelism available. Each column of L depends on the immediately preceding one, and thus they must be computed sequentially. This behavior is typical of orderings that minimize the bandwidth of a sparse matrix: they tend to inhibit rather than enhance large grain parallelism in the factorization

Figure 3 Reordered matrix elimination tree

Figure 2 shows the same one dimensional mesh with the nodes reordered by minimum degree algorithm. We see in Figure 2 that L suffers no fill in the new ordering, and the elimination tree now shows some large grain parallelism. In particular, columns 1 and 2 can be computed simultaneously, then columns 3 and 4, and so on. This twofold parallelism reduces the height (roughly the parallel completion time) by approximately a factor of two.

Figure 4 Reordering with fill

Figure 3 shows the same matrix reordered by nested dissection (a divide and conquer approach). This gives rise to some fill but also gives greater parallelism.

# 6.	Parallel Algorithm

The parallel algorithm for the sparse factorization is given below. Before that some definitions:

Map[j]: if J is a set of column numbers, map[J ] will denote the set of processors containing the given columns. The columns are divided equally among the processors.

Struct(M*j) = { k > j | mkj ≠ 0} i.e it is the sparsity structure of column j of the strict lower triangle of M.

The pseudo code the algorithm is given below. The algorithm given below runs on each processor, with each responsible for its own subset, mycols, of columns.

for j ε mycols if j is a leaf node in T(A) cdiv(j) send L*j to processors in map(Struct(L*j )) mycols = mycols – {j} while mycols ≠ φ receive any column of L, say L*k for j ε mycols ∩ Struct(L*k) cmod(j, k) if column j requires no more cmods cdiv[j] send L*j to processors in map(Struct(L*j )) mycols = mycols – {j}

Figure 5  Parallel algorithm

In the algorithm above, any processor that owns a column of L corresponding to a leaf node of the elimination tree can complete it immediately merely by performing the necessary cdiv operation, since such a column depends on no prior columns. The resulting factor columns are then broadcast (fanned out) to all other processors that will need them to update columns that they own. The remainder of the algorithm is then driven by the arrival of factor columns, as each processor goes into a loop in which it receives and applies successive factor columns, in whatever order they may arrive, to whatever columns remain to be processed. When the modifications of a given column have been completed, then the cdiv operation is done, the resulting factor column is broadcast as before, and the process continues until all columns of L have been computed.

This algorithm potentially exploits both the large grain parallelism characterized by concurrent cdivs and the medium grain parallelism characterized by concurrent cmods.

# 7.	Storage Schema

In order to take the advantage of the great number of zero in the original matrix, we use a special storage schema. The main goal is to represent only the non-zero elements in the matrix and to be able to perform common matrix computations.

The schema is called Compressed Sparse Row (CSR) format. The data structure consists of three arrays. The first array contains real values of all the non-zero elements in the matrix. The second array is an integer array containing the corresponding column indices. The third array is also an integer array containing the pointer to the first element in each row in the first two arrays. Below is an example of CSR storage format.

Figure 6 Matrix A

First array containing real value

Second array containing column indices

Third array containing pointers to first element of each row

Figure 7 CSR format for Matrix A

This schema is probably the most popular one for storing sparse matrices because it is often more use for performing typical computations. There are a number of variations for the Compressed Sparse Row format. The most obvious variation is storing the columns instead of the rows. The corresponding scheme is known as the Compressed Sparse Column (CSC) scheme. For symmetric matrices that we deal with in the project, it is the same to store in both way.

# 8.	Level of Fill

During cholesky factorization, the modification of a column not only changes the non-zero entries in the target column, but may also introduce new non-zero entries in the target column. Thus the Cholesky factor L may have additional non-zeros, called fill, in locations that were zero in the original matrix. In determining the storage requirements and computational work, these new non-zeros that the matrix gains during the factorization are equally important as the non-zeros in the original matrix.

Because with CSC schema, we do not store all the matrix entries in the memory, we need to choose the elements that should be put in the arrays. It is clear that all the non-elements and fills should be stored. The problem is that we need to tell whether a zero element is a fill or not. Obviously, it is impossible to evaluate a zero element before computation. However, the key point is that before do the numerical computation for the matrix. We do a symbolic computation for all the entries in advance. The symbolic computation is done regardless of the real value of the entries. It only computes the level of fill of each entry. A level of fill is attributed to each entry and is processed by the same algorithm that is used to compute Cholesky factor L---Figure 1: Cholesky factorization. Instead of updating the real value of the element, we update the level of fill of the element. Initially, all the non-zero elements have a level of fill zero, while all other zero elements have a infinite level of fill. The pseudocode of the algorithm is described below:

for i = 2, n for k = 1,i - 1 for j =k+1, n levelij=minlevelij,levelik+levelkj+1

Figure 8 Level of Fill computation algorithm

In Cholesky factorization, we need to specify the level of fill Levelfill we want in the result matrix. After the fill level computation, we consider all the elements with level value not larger than Levelfill as fills and thus store these elements in CSC format for further numerical computation.

If the level of fill is implemented serially, the running time of this will dominate that of computing the factorization of the coefficient matrix. Therefore, the level of fill computation is implemented in parallel. The parallel version is the same as numerical computation. Thus, doing symbolic computation in advance tom get the locations of fills will not bring a side-effect to the running time of Cholesky factorization.

# 9.	Reordering

Reordering a matrix A is to permutate the rows, and/or columns, to form another matrix A’.  If the other matrices in the linear equation Ax=b are permutated in the same manner, the solution produced for the vector x is the same.  Typically, the permutation matrix P is just the identity matrix with its columns permutated.  Since in this project, we are considering Cholesky preconditioners which are symmetric, we wish to maintain the symmetry of the matrix.  Therefore, the same permutation is applied to both the rows and the columns, which can be given as follows:

A’=PAPT

The algorithms usually provide the permutation in the form of a permutation array iperm.  This contains the permutated order of the old rows/columns with respect to the new rows/columns.

The objectives for reordering a matrix include the following:

1.	To reduce the amount of fill in the preconditioner.

During computation, if the level of fill desired is not zero, the value of the fill has to be calculated.  Moreover, since a compressed form of the sparse matrix is used, updating the matrix (or calculating the fill positions and modifying the storage structures to accommodate them) takes time and storage space.

2.	To make the matrix work better with parallel algorithms.

The elimination tree constructed after reordering the matrix may become more “bushy”, i.e. have a shorter height and higher fan out (child nodes per node).

Some reordering algorithms include:

-	CutHill-McKee Reordering

-	Reverse CutHill-McKee Reordering

-	Nested Dissection

-	Minimum Degree

-	Approximate Minimum Degree

In the project, we have implemented the CutHill-McKee (CM) and Reverse CutHill-McKee (RCM) algorithms to test out the first objective.  However, we have included test results for performance comparison on time taken with and without reordering for a fill level of zero to see if the second objective has been met somewhat.  The reordering implementation is serial as it is fast – the factorization process takes up a much more significant portion of the computation time.  In fact, reordering takes less than half a second a matrix of size 2000x2000 with a sparsity of 0.05.

# 10.	CutHill-McKee/Reverse CutHill-McKee Reordering

The CutHill-McKee reordering algorithm on a graph G with no disconnected components can be given as follows:

1.	Choose an arbitrary node in G, r.

2.	Add r to a queue q.

3.	Mark r as being visited.

4.	Allocate an empty array iperm with size the number of nodes in G.  Set next=0.

5.	Remove the first node k from q.

6.	Add its children, if they have not been visited, to q, in ascending order of their degree in G.

7.	Mark all those children added as visited.

8.	iperm[next]=k.  next=next+1.

9.	If q is not empty, go to step 3.

10.	iperm contains the permutation needed to be performed on matrix A.

In short, a breadth first search is done on G with the nodes ordered by degree.  The order in which the nodes are visited are recorded into the array iperm.  In general, the graph representation of a matrix G(A) does not have only a single connected component.  In this case, all the connected components are simply traversed one at a time using the above algorithm and the traversed order recorded in iperm.

The time taken to execute the above algorithm depends on the number of nodes in the graph, which is n, the number of rows/columns of the matrix A.  In addition, the algorithm takes O(n) time, since each node is traversed exactly once, and if the nodes if the graph are placed in an array, the array need only be scanned once.  Thus it is quite fast.

To obtain the permutation array for Reverse CutHill-McKee, values in the iperm array obtained above are reversed in order.  That is, if iperm contains values {i1, i2, …, in}, the permutation array for RCM is {in, in-1, …, i1}.  It has been observed that the RCM algorithm produces much less fill than CM.

In step 1 of the algorithm, an arbitrary node is chosen to start the reordering.  The choice of the start node affects the effectiveness of the reordering.  One way of obtaining a good start node is to find the peripheral node of the graph.

# 11.	Peripheral Nodes

The objective is to find two nodes in a graph G which are at a maximum distance apart.  That is, the shortest distance between the two nodes are maximal in G.  A few terms are defined below to help define the above concept more clearly.

The eccentricity of a node x in a graph G is as follows:

l(x) = max{d(x, y) | y  G}

where d(x, y) is the shortest distance/path between nodes x and y in the graph.  To get l(x), the shortest path of all nodes in the graph to x can be obtained, and the maximum of all these would be the eccentricity of x.

The diameter of G is given as follows:

dia(G) = max{l(x) | x  G}

A node x in G is a peripheral node if its eccentricity is equal to the diameter of G.  Since the best known algorithm (according to George Lui et al) has a time complexity bound of O(|G||E|), where E is the set of edges in the graph G, an efficient heuristic algorithm is implemented to find nodes of high eccentricity instead.  The nodes found by the algorithm are termed as pseudo-peripheral nodes.

Before the algorithm for finding pseudo-peripheral nodes is given, the level structure of a graph G rooted at a node r is given as follows:

L(r) = {L0(r), L1(r), …, Ll(r)(r)}

where

Lo(r) = {r}

L1(r) = Adj(L0(r))

Li(r) = Adj(Li-1(r) - Li-2(r)), i = 2, 3, …, l(r)

Similarly, the level structure can be obtained using a Breadth First Search.  Each level in the graph is simply the set of nodes produced at each level of the traversed tree.

The algorithm for finding pseudo-peripheral nodes in a graph G with a single connected component is given as follows:

1.	Choose an arbitrary node r in G.

2.	Construct the level structure of r, L(r).

3.	Choose a node x in the last level of L(r), Ll(r)(r) with minimum degree.

4.	Construct the level structure of x.

5.	If l(x) > l(r), set r to be x, and go to step 3.

6.	The node x is a pseudo-peripheral node.

Figure 9  Pseudo-peripheral node example

The above diagram is an example of the described algorithm.  In the first figure, the node r is arbitrarily chosen.  The eccentricity of this is 2. x is the node r in the second figure with eccentricity 4.  Since l(x) > l(r), the algorithm proceeds to set x as the new r as in the second figure.  The new x found is the node r in the third diagram.  Since l(x) = 4 = l(r), the algorithm terminates, and the node r in the third diagram is the peripheral node found.

# 12.	Matrix Diagrams

The following are diagrams of reordering that is performed on a 250x250 matrix with a sparsity of 0.05.  The output from the program is read into MatLab and the spy() function is used to produce the diagrams below.

Figure 10 Original Matrix n=250 sparsity=0.05

The above is the original matrix that has not undergone any reordering.

Figure 11 CM reordering

The above is the matrix that has been reordered using the CutHill-McKee reordering algorithm.

Figure 12 RCM reordering

The above is the matrix that has been reordered using the Reverse CutHill-McKee reordering algorithm.

Figure 13 CM reordering using pseudo-peripheral nodes

The above is the matrix that has been reordered using the CutHill-McKee reordering algorithm with the start nodes as the pseudo-peripheral nodes found.

Figure 14 RCM reordering using pseudo-peripheral nodes

The above is the matrix that has been reordered using the Reverse CutHill-McKee reordering algorithm with the start nodes as the pseudo-peripheral nodes found.

Figure 15 RCM using MatLab

The above is the matrix that has been reordered by using MatLab’s Reverse CutHill-McKee symrcm() function.

# 13.	Test Results

The figures below are taken from programs running on beowulf.lcs.mit.edu.  The time taken values are the values of the maximum time taken for our MPI implementation.  That is, since all nodes take different time to complete the factorization, we take the maximum time.  The time taken is the time taken for computing the factorization.  It does not include the time taken to read in the input matrix, but it does include the time taken to reorder and calculate the level of fill.

Figure 16 Graph of matrix size against time taken

Observations:

1.	The time taken by our serial implementation grows much faster than any of the other implementations.  This shows the importance of parallelism for the factorization process.

2.	The time taken for factorization is slightly better than that with no reordering.  This shows that the reordering improved the running time slightly.

3.	However, MatLab*P still executes much faster than our implementation.  This could be due to a number of factors:

a.	Our implementation does not handle load balancing well.  The columns are divided equally into strips and assigned to each processor.  This causes some processors to be much more heavily loaded than others.  A better way would be to use the elimination tree to assign the columns.

b.	MatLab*P may be using a better storage method than our implementation.  For CSR format, an element access requires 3 array references.

c.	MatLab*P may be using a faster algorithm.

Figure 17 Graph of Level of Fill against Fill produced

Observations:

1.	Reordering helps to reduce significantly the amount of fill produced.

2.	There doesn’t seem to be much of a difference whether the pseudo-peripheral nodes are used as the root, although it does produce less fill than just RCM at lower levels of fill.

Figure 18 Graph of Level of Fill against Time taken

Observations:

1.	Time taken with reordering is significantly better than without reordering.

2.	Although finding the pseudo-peripheral nodes for use as the root slightly improved the fill produced in the previous graph, in this case, the time taken is slightly more than just using RCM.

Figure 19 Graph of matrix size against time taken

Observations:

1.	The graph produced is almost the same as the graph in Figure 16.

2.	We note that CM produces the best result overall.  The load distribution seems to be better for CM since the reordered values do not seem to cluster on only a few nodes.

Figure 20 Number of nodes against Time Taken

Observations:

1.	CM is much more scalable than any of the others.  Even when run serially, CM is still better than without reordering, unlike RCM.

2.	Again, find pseudo-peripheral nodes do not seem to make much of a difference.

Figure 21 Level of Fill against time taken

Observations:

1.	Reordering helps to reduce significantly the time taken when fill increases.

2.	CM takes less time than RCM.

Figure 22 Level of Fill against Fill Produced

Observations:

1.	Reordering helps to reduce significantly the amount of fill produced.

2.	CM produces more fill than RCM.

# 14.	Future Work

Due to time constrains, we would have liked to do the following, but have not done them:

1.	Load Balancing

2.	Testing the generated preconditioner with an actual solver such as Conjugate Gradient

In addition, the following could be done to further explore preconditioning:

1.	Effects of storage formats

2.	Other reordering methods such as Minimum Degree, Approximate Minimum Degree, Nested Dissection, etc

3.	Other methods for computing the preconditioner in parallel

# Acknowledgements

We would like to thank Professor John Gilbert for his guidance throughout the course of the project, as well as Ron Choy for his unwaivering dedication to keeping Beowulf up and running.

# Appendix A: Running the submitted programs

The submitted files are:

1.	gen.m

2.	generate.m

3.	MPI_Cholesky.c

4.	cholesky_serial.c

5.	m.zip containing matrices of various sizes

Running the Cholesky factorization:

The input matrix should be stored in matrix.txt file, in the same directory as the executable.

To build the serial version of the Cholesky factorization, use the following command:

gcc –O –o cholesky_serial cholesky_serial.c -lm

To run the serial version of the Cholesky factorization use the following commands:

cholesky_serial

To build the parallel version of the Cholesky factorization, use the following command:

mpicc –O –o MPI_Cholesky MPI_Cholesky.c -lm

To run the parallel version of the Cholesky factorization use the following commands:

mpirun –np p n1-8 MPI_Cholesky <[0|1] [0|1|2|3|4|5|6] [x]>

where 	p is the number of processors to be used,

x is the level of fill desired (default = 0)

the first option is for debug printout (0 = no printout, 1 = printout)

the second option is for reordering

(0 – no reordering, 1 – RCM, 2 – CM, 3 – RCM with pseudo-peripheral nodes, 4 - CM with pseudo-peripheral nodes, 5 - RCM with pseudo-peripheral nodes with sorting of components, 6 - CM with pseudo-peripheral nodes with sorting of components)

For example,

mpirun –np 8 n1-8 MPI_Cholesky

to run the program without printout, no reordering and zero levels of fill on 8 nodes

mpirun –np 12 n1-8 MPI_Cholesky 1 1 2

to run the program with debug printout, RCM reordering and 2 levels of fill on 12 nodes

Generating the input matrix:

generate.m was used to generate the matrices in the old format where matrix.txt is the full listing of all elements in the matrix with zero values included.  This format is used for matrices as input to the serial version of the program.

For example,

generate(1000, 0.05, 0)

will cause MatLab to generate a 1000x1000 matrix with sparsity 0.05 and run MatLab’s Cholesky factorization on it with fill level 0.  The generated matrix will be saved in the file matrix.txt.

gen.m is now used to generate the new matrix format where only non-zero values are stored in matrix.txt.  However, before the stored matrix can be used in the parallel MPI program, the last line generated by MatLab has to be placed as the first line.

For example,

gen(1000, 0.05, 0)

will cause MatLab to generate a 1000x1000 matrix with sparsity 0.05 and run MatLab’s Cholesky factorization on it with fill level 0.  The generated matrix will be saved in the file matrix.txt.

# Appendix B: Work Division

The figure below shows the work division among different members of the team.

|  | Aditya | Wu Yong Hua | Low Wai Chong |
| --- | --- | --- | --- |
| Read Input |  |  |  |
| T(A), G(A) |  |  |  |
| Cholesky |  |  |  |
| Reordering |  |  |  |
| Pseudo-peripheral Nodes |  |  |  |
| Fill Levels |  |  |  |
| Debug |  |  |  |
| Serial Code |  |  |  |
| MatLab Code |  |  |  |
| Reading & Design |  |  |  |
| Documentation |  |  |  |
| Data Collection |  |  |  |

Figure 23 Table of work division

# References

[GEO94]	Alan George, Joseph Liu, Esmond Ng, “Computer Solution of Sparse Linear Systems”, January 5, 1994

[DEM93]	James W. Demmel, Michael T. Heath, Henk A. van der Vorst, “Parallel Numerical Linear Algebra”, November 4, 1993

[SAA00]	Yousef Saad, “Iterative Methods for Sparse Linear Systems”, January 3rd, 2000