SMA5505 – Applied Parallel Computing

Parallel Implementation of

Full-motion Video Information Retrieval

By

Ameya Virkar (smap1068)

Qiu Qiang (smap1053)

Qian Haichun (smap1051)

TABLE OF CONTENTS

Abstract	3

1	Introduction	3

2	Video Information Retrieval	4

2.1	Shot Boundary Detection	4

2.1.1	Histogram Comparison	5

2.1.2	Twin-comparison Method	6

2.2	Video Indexing and Summarization	7

3	Parallel Video Information Retrieval	7

3.1	An Overview of MPEG1	8

3.2	Parallel MPEG1 Video Decoding	9

3.3	Parallel Histogram Difference Metric Calculation	9

3.4	Structure of Partial Frame	10

3.5	Computing Threshold Value - Tb	11

3.6	Implementing Twin-comparison	12

3.7	Marking key frames	13

4	Performance Analysis	13

4.1	Focus	14

4.2	Experimental Setup	14

4.3	Using Distributed Memory Approach	15

4.4	Results	15

4.5	Issues related to Speed Up	16

4.6	Reasons behind the speed up	17

5	Executing the program	18

6	Future Work	18

7	Conclusion	19

8	References	19

9	Acknowledgements	19

10    Individual tasks	20

# Abstract

A video information retrieval system could enable great use of video sources by providing easy and effective organization and retrieval of information from those sources. However due to the huge computation involved, such tasks normally take significant time. The goal of the project is to present a parallel implementation of full-motion video information retrieval system. This parallel system aims at partitioning the video into meaningful segments by detecting segment boundaries and retrieving key frames in the video. With such parallel video information retrieval system, video summarization and indexing could be even achieved in real-time.

# Introduction

Video is a very important and common medium for applications as varied as education, broadcasting and publishing. The value of video is partially due to the fact that significant information about the major aspects of the world can only be successfully managed when presented in a time-varying manner. There has been a great need for effective organization of the videos using indexing and also information retrieval from them. Unfortunately the intuitions obtained by studying the indexing and searching from the non-text media cannot be applied to videos. The growing interest in digital video has lead to immense research in the field of video indexing.

Today there do exist some good algorithms to enable automated video indexing and summarization by detecting and isolating the meaningful segments in any video source, i.e., twin-comparison method [1].   Normally, those algorithms will use color histogram of the each frame for the shot boundary detection to partition the video into meaningful segments. However, due to the fact that video indexing and summarization requires the sequential scan of the video source, during which also various operations have to be performed such as frame decoding, histogram calculation, large computations are involved and the amount of computation increases with the size of the video under consideration. This makes parallel video information retrieval a very promising technology. The motivation behind this project is to implement the computation across different machines running in parallel and thus offer a speedup in the entire process. With the power of parallel computing, if we could reduce the time needed for those tasks for video information retrieval to a sufficient short period, lots of wonderful things could become possible. For example, before watching an online video, people may like to just request a real-time video indexing and summarization; instead of spending a long time and a large bandwidth to download everything, now we may just enjoy the desired portion of the video.

This report is organized as follows. Section 2 introduces the video information retrieval algorithm. The parallel implementation details are given in Section 3. The performance analysis is presented in Section 4. Section 5 and 6 describe the usage and future work. Section 7 concludes this report.

# Video Information Retrieval

In order to enable video information retrieval, we need to perform the following task:

- Segmentation: We need to parse a given video into its elemental units. These units are known as shots.

- Video indexing and summarization: We then need to store all the elemental units with significant content in a database, which allows efficient indexing and effective retrieval.

- Interactive Tools: Access tools are required to interact and view this data. These include presentation tools, editing tools and browsing tools.

## Shot Boundary Detection

Our basic motivation is to make video as a first-class data type on the computer. For this purpose we need to define the elemental units of video. These units are known as shots. A shot is an image sequence, which presents continuous action that appears to be from a single operation of the camera. A camera break is the simplest transition between two shots. More sophisticated techniques include dissolve, wipe, fade-in and fade-out. Such special effects involve much more gradual changes between consecutive frames than does a camera break. We have adopted twin-comparison technique to detect both camera break and gradual transition break.

### Histogram Comparison

This idea here is to use color histograms of pixels in a frame as a feature to detect shot boundaries. A frame contents represented in RGB domain (R, G, B) is linearly re-quantized into N (N=159) levels LUV domain (L, U, V) by the following conversion equations:

L =  116(Y/Yn)1/3-16

U = 13L(u’-u’n)

V = 13L(v’-v’n)

Where

u’ = 4X/(X+15Y+3Z)

v’ = 9Y/(X+15Y+3Z)

v’n = 4Xn/(Xn+15Yn+3Zn)

X= 0.607R + 0.174G + 0.200B

Y = 0.299R + 0.587G + 0.114B

Z = 0.000R + 0.066G + 1.117B

Each frame in LUV domain is represented as a vector H of size N where H[i] is the number of pixels having the intensity level i and scaled by the total number of pixels. The histogram difference between frame I and J is:

SD(I,J)=k=0N−1|HI[k]−HJ[k]|

The assumption is for the same background and same objects, there is little change in the histogram.  If the difference exceeds a threshold, i.e. SDi > Tb, then a short boundary is detected. Appropriate selection of threshold is very important. If there is no shot boundary, then the frame-to-frame difference is due to three types of noise: 1.) Video production equipment noise 2.) Digitization noise 3.) Non-stillness of object noise assuming that such a noise is Gaussian in nature. Therefore, frame-to-frame noise is the sum of Gaussian noise, camera-breaks, gradual transitions and camera movement. Let u and  be the mean and standard deviation of the frame-to-frame differences of a video.  The threshold Tb can be chosen this way:

Tb = u + 

Where  is chosen as 5 in our implementation.

### Twin-comparison Method

Many edit effects are gradual transitions for aesthetic and artistic reasons. For such shot boundary, a single threshold mentioned above will not help. A twin-comparison approach can be employed for detecting such transitions. The core idea is the use of two thresholds Tb and Ts, instead of one global threshold. Tb is used for detecting camera-breaks that are abrupt transitions. Ts is used for special effects detection that includes usually gradual transitions.

The twin-comparison algorithm is as following:

- Compute SDi for all frame sin the video sequence.

- Mark camera breaks where Sdi > Tb

- Mark potential gradual transition subsequences GT = {[Fs, Fe]} of the video wherever SDi > Ts for Fs ≤ i ≤ Fe

- For each potential gradual transition [Fs, Fe], accumulate the frame-to-frame difference:

AC =∑ SDi

- If AC > Tb, then declare [Fs, Fe] as a gradual transition.

## Video Indexing and Summarization

Shots are the basic units of manipulation but there is a need to get a handle on to the content of a video for video indexing and summarization. Key frames serve to capture the significant content of the video. If the entire video is uniformly sampled for summarization, either the important content may be missed or there may be redundant information. The best way is to do shot boundary detection first and pick the frames that could most represent the shot as following:

- Do shot boundary detection

- Compute the normalized histogram for every frame in shot k

- For every bin of the histogram, compute the average frequency of pixels

- Obtain the average histogram of shot k using the average frequencies. Normalize it.

- Compare the normalized histogram of each frame of shot k with it average histogram

- Pick the most similar frame as the key frame.

The key frame retrieval step could be repeated for more representative frames for each shot. Now the video sequence is possible to be partitioned into meaningful segments and summarized by those representative frames.

# Parallel Video Information Retrieval

This section presents the design methodology for parallel implementation of a video information retrieval system. There are several possible levels of parallel processing for video:  pixel level, frame level or combination of both. This parallel video information retrieval system was implemented in C with MPI functions.

## An Overview of MPEG1

MPEG1 video stream is used as the source video for information retrieval. MPEG1 divides the compressed video frames into three types: I pictures, P pictures and B pictures (Figure 1). I pictures, the intracoded pictures, are encoded without reference to other frames using only the spatial correlation within a frame. P pictures, the predictive coded pictures, are encoded using motion compensated prediction from a past I picture or P picture. The bidirectionally-predictive coded pictures, or B pictures, require both past and future reference frames (I pictures or P pictures) for motion compensation.

Figure 1: An example of a video sequence with the I, P or B picture assigned to each frame.

An MPEG1 encoder needs to specify which frames of the input video sequence are to be compressed as I, P or B picture. This pattern of I, P and B pictures usually repeats itself throughout the video sequence. An example of a pattern of frames is IBBPBBPBB, where I, P and B indicate I picture, a P picture and a B picture, respectively. Hence an input sequence could be:

I1 B2 B 3 P4 B5 B6 P7 B8 B9 I10 B11 B12 P13 B14 B15 P16 B17 B18 I19

The order of the frames in the output sequence is rearranged according to the order in which an MPEG decoder can decompress the frames with minimum frame buffering (a maximum of three frame buffers). Hence, at the output of the encoder for the above sequence, the frame order has the form

I1 P4 B 2 B3 P7 B5 B6 I10 B8 B9 P13 B11 B12 P16 B14 B15 I19 B17 B18

In an MPEG encoded sequence, several consecutive frames are combined to form a structure known as a group of pictures (GOP). The first picture in a GOP must be an I picture (a GOP can contain one or more I pictures). The existence of GOPs facilitates the implementation of features such as random access, fast forward, or fast and normal reverse playback.

## Parallel MPEG1 Video Decoding

Frame level parallel processing is adopted for video decoding phase. However, both P and B frames are non-intracoded i.e. past or future reference frames (I pictures or P pictures) are required for motion compensation during decoding. In order to avoid the communication overhead on exchanging the depended frame information, the MPEG1 video is parallel decoded in the unit of GOP. For simplicity, each GOP is assumed to be encoded with a P frame at the end; otherwise, if it is end with a B frame, the starting I frame of the next GOP have to be included to make it self-contained for decoding. Now each GOP contains all the information for independently decoding all the frames in the GOP.  This makes it possible that each process could load in one GOP to perform the frame decoding without any communication with other process.

## Parallel Histogram Difference Metric Calculation

Pixel level parallel processing is adopted for histogram difference metric calculation phase.  The histogram of each frame is maintained as a distributed matrix. Let the number of the process by nProc, the rank of the process represented as i and the LUV level be N. At the end of the decoding of each GOP, the process i will only keep the histogram for (i*N/nProc)th bin and the subsequent  N/nProc bins, and pass the rest histogram to the corresponding process, e.g. the histogram for (j*N/nProc)th bin and the subsequent  N/nProc bins to process j.  A partial histogram difference will be computed parallel for the corresponding portion of histogram at each process. At the end of each computation, the partial histogram difference from all processes could be summed up.

## Structure of Partial Frame

For further stages of computation, we save a two-dimension-array of frame structure in each processor. For each frame structure, the following data is included:

- ImageName: The name of the individual frame or image.

- Seqno: This field indicates the sequence number of the frame in the video.

- Sd: This field is used to indicate the histogram difference of the current frame with the previous frame, and is the sum of the partial Sd values from all the processors.

- Partial Sd: The partial sd values stored by each processor are available from this field in the array. The values in this field are used for the computation of sd.

- SeqMark: This field indicates the frame type. Different values of the seqmark are ‘b’ for “camera break”, ‘s’ for start of gradual transition and ‘e’ means end of gradual transition.

- IsKey: This field indicates whether the frame is key frame or not.

- hist: This handles the partial histogram that is stored in every processor. It is a single dimensional array of length N/nProc.

## Computing Threshold Value - Tb

Figure 2: Communication in computation of Threshold Value Tb

As mentioned in the earlier part, Tb can be calculated with following formula:

Tb = u + 

In above formula, u is the mean of the histogram differences (sd) and  is the deviation of histogram differences (sd). After computing the histogram differences, Tb can be calculated as follows:

- To parallel the calculation of u, the sum of histogram differences is first computed. For this we create a binary tree for adding these numbers. The summation takes exactly log2P steps and involves exchange of data between the child-parent processors.

- The value of mean is computed at the root node and is passed to each processor, i.e. by passing the sum back down to the leaves of the binary tree.

- Next the mean deviation of histogram difference for the frames is calculated. The root node receives the values from other processors again using the binary tree structure. The root node computes the threshold value using the mean and deviation values and passes it back to the leaves of the binary tree.

The parallel scheme of this stage is based on frame level implementation, which means that the frames are distributed among multiple processors.

## Implementing Twin-comparison

The camera break and gradual transition break in frames are detected using the twin-comparison algorithm. The twin comparison method uses two threshold values namely Tb and Ts. The camera breaks are detected by comparing the values of the histogram difference for each frame with Tb. For camera breaks, the histogram difference exceeds the value of Tb. Therefore, it becomes trivial to detect a camera break and the process involves scanning the frames and marking those frames that have the histogram difference to be greater than the value of threshold Tb.

The next stage is the detection of gradual transitions. The gradual transitions involve slow motion videos or intermixing of frames. Such effects have to be handled by using the other threshold. The frames that are a part of the gradual transitions have their histogram difference values between the threshold i.e. Ts < sd < Tb. The first frame exhibiting this property is marked as the potential start frame and the subsequent frames are considered a part of the gradual transition. Frames are compared with subsequent frames and accumulated comparison metric is computed. The accumulated comparison metric is the summation of histogram differences of all the frames in the gradual transition. The end frame of the transition is detected when the difference between the consecutive frames decreases to less than Ts while the accumulated comparison has increased to a value larger than Tb. Note that the accumulated difference is only computed when the difference between the consecutive frames exceeds Ts. If the consecutive difference value drops below Ts before the accumulated comparison value exceeds Tb, then the potential start point is dropped and the search continues for other gradual transitions.

In a parallel implementation, it is possible that the start and end frames of a gradual transition are handled by different processes. Also once the start and end frames have been marked, it is required to compare the accumulated comparison value with that of Tb. Therefore, in parallel implementation, the value of the accumulated comparison has to be transferred between the adjoining processes. Also whenever the start and the end frame positions are to be dropped because of lower accumulated comparison value, then the adjoining processes have to be informed to demark the start frame. The parallel implementation thus involves some overhead of communication in the case of identifying frames in gradual transitions.  Also the parallel scheme of this stage is based on frame level implementation.

## Marking key frames

The parallel implementation of mark-key-frame is based pixel layer.

- For each processor, select all the camera breaks in the whole frames. Because the information about camera break is exchanged among the processors during twin-comparison, each processor holds the information about the positions of camera break happen.

- For the frames between the two camera breaks, the partial average histogram is calculated. Also the partial similarity is computed between the histogram of each frame and the partial average histogram.

- Through the communication among all processors, the overall histogram similarity can be computed for all the frames between the camera breaks. Then the most similar frame is chosen as the key frame.

# Performance Analysis

This section is to identify the performance monitoring and modeling requirements for the Parallel Implementation of Full motion video information retrieval project.  Our initial focus is on the MPEG video files having significant scene changes and camera breaks. Future works entails extending the functioning for Real Video files. In terms of requirements, this section attempts to address the following issues:

- How much speed up did you get?

- Explanation on the speedups

## Focus

The serial version of the implementation was found to take considerably more time to get the results. Also it was noted that the memory requirements of the serial version increased for larger videos.

So with these two main difficulties in mind, the focus of the implementation has been on improving the speed and the memory utilization of the program. The focus also has been on load balancing and reducing communication overhead.

## Experimental Setup

The setup used for experimental analysis is important for understanding the environment under which the program was tried and tested. We have used BEOWULF cluster at MIT (beowulf.lcs.mit.edu) for testing the programs.

The program was run on a Beowulf cluster having the following specifications:

- Nodes: 9 (1 front-end + 8 compute)

- Processor: 2 x Athlon MP 1800+ (1.53GHz) on each node

- Memory: 1 GB PC2100 DDR EEC Registered on each node

- Disks: 2 x IBM 120GXP 40GB on front-end, 1 on compute nodes.

- Network: Nodes are connected by 3 fast Ethernet switches (100 Mbps), joined by channel bonding, yielding a total of 300Mbps bandwidth

- Peak performance: 3 FPU, 1.53GHz processors, with peak of 4.59GFlops. 4.59 x 18 giving 82.62 GFlops peak for the Beowulf. (This is the peak for SINGLE precision floating point calculations).

The videos chosen were MPEG1 videos with sufficient camera breaks in them.

## Using Distributed Memory Approach

Message passing is a paradigm used widely on certain classes of parallel machines, especially those with distributed memory. The main advantages of using message-passing standard are portability and ease-of-use. In a distributed memory communication environment in which the higher level routines and/or abstractions are build upon lower level message passing routines the benefits of message passing are particularly apparent.

The basic issues involved in cluster and distributed memory parallel computing has been the communication. The parallel program completes a complex computation by sharing the workload amongst the different running processes and thus is able to achieve results more quickly than one processor.

We have implemented the parallel version of the serial code. The parallel version achieves node level parallelism by using the distributed memory approach. The Message Passing Interface or MPI is a library of functions and macros that can be used in C language. MPI has been for use in programs to exploit the existence of multiple processors by message passing.

## Results

The program has been tested on different number of processors (2, 4, 8) and the results obtained in each case have been recorded and shown below. The results are in accordance with the expectation of writing a parallel implementation i.e. lesser time and better memory utilization than the serial implementation. The speed up achieved is evident from the results obtained. The performance improvement has been noted in terms of both the time required to calculate the various details of the video plus also the memory utilization across the distributed network.

The results of the experiments have been listed below:

| No. Processors | Total Time taken (sec) |
| --- | --- |
| 1 (Serial Version) | 104.785 |
| 2 | 73.964 |
| 4 | 30.502 |
| 8 | 11.683 |

The graph showing the improvement in the speed up is given below:

Figure 3: Speed up with increasing number of processors

## Issues related to Speed Up

There are several issues that affect the speed up in the implementation. These issues are given below:

- Large memory requirements: The program has large memory requirements. Memory is consumed during decoding the MPEG video and also during the calculation of the histogram values.

- Weakness in the algorithm: The threshold values used in the detection of camera breaks require histogram values of all the frames in the video. One way to record the histogram values is to store them in a file and make the file accessible to all. But we found that the Input/Output involved in retrieving values from the file degraded the performance of the program. So it was decided to keep the histogram values in memory.

In order to avoid storing all the histogram values simultaneously, we can process (say) 100 frames of the video and based on their histogram characteristics guess the value of threshold for these frames. In this way, it would be necessary to keep only few histogram values in memory. We have decided to keep this as our future work on the project.

## Reasons behind the speed up

As stated before the memory requirements of the program are more because decoding of video and histogram calculation require extra memory space.

When one processor is handling the implementation, it encounters the additional burden of managing the entire space allocated to the histograms. It is quite possible that the histogram values stored in the main memory are regularly swapped with the disk storage.

In the case of distributed memory approach for parallel implementation, the memory requirements are divided into the various participating nodes. The approach used in the parallel implementation is the divide-and-conquer strategy. The computation required for finding camera breaks etc. are handled in parallel by the nodes. This division of work solves the problem faster and speed up is enhanced by carefully balancing the workload and communication overhead. Large memories and multiple processors are also helpful in improving the response times in such a system.

# Executing the program

The source code of the parallel implementation has been submitted with the project report. The entire source code is available in the form of a single tar file. Untar the tar file using packages like WinZip under Windows or GTAR under Unix. The directory contents after extraction of the files:

- color.c  color.h: 	RGB to LUV convertion tools

- videosegment.c: 	Main program for video information retrieval

- Makefile: 		Makefile

- data directory: 	LUV Color simularity matrix

- include directory: 	Header file for frame decoding

- lib directory: 		Library used for frame decoding

For using the program, the following two steps are to be carried out:

- Compile the program using the “make” command

- Execute “videosegment“ with the video frames as the input

# Future Work

The future work would include the following enhancements:

- We plan to extend the implementation on Real Media format video also.

- The automated indexing and summarization try to be enabled for real-time video. The threshold values using in the program are calculated using the histogram values. We plan to calculate the threshold values without storing all the histogram values simultaneously. For this, we would process certain frames in each iteration and guess the threshold values depending on the characteristics of the frames in the video.

- An interactive tool will be developed to allow the user to browse and view through the video. For example, the tool would allow the user to move to the first camera break in the video etc.

- Lastly we would still want to improve the performance of the program in terms of both speed up and memory utilization.

# Conclusion

In this project, a parallel implementation of full-motion video information retrieval system was presented. Automated video indexing and summarization was enabled by partitioning the video into meaningful segments by detecting segment boundaries and retrieving key frames in the video. From the performance analysis, we could observe that almost a linear speedup could be achieved with increasing the number of process employed under our implementation. Our parallel video information retrieval system could provide a fast, easy and effective way to organize and retrieve information from video sources.

# References

- HongJiang Zhang, Atreyi Kankanhalli, Stephen W. Smoliar; Automatic partitioning of full-motion video, Multimedia Systems (1993) Volume 1, Pages 10-28.

# Acknowledgements

We would to express our sincere thanks to Prof. Alan Edelman and Prof. Hsu Wen Jing for their guidance throughout the project duration. We would like to thank them for approving the project proposal and also for providing us an opportunity to implement the project.

# Individual tasks

The parallel implementation methodology was designed by together.

The initial serial implementation, which was used as both framework and benchmark for the later parallel implementation, was done by Qiu Qiang.  He also took over the parallel video decoding, parallel histogram difference metric calculation and extended the progress write-ups submitted by group members to the final document.

Ameya Virkar has handled the parallel implementation of the threshold calculations; parallel implementation of video (camera) break detection and twin comparison technique and also part of parallel key frame retrieval. He also collected the data and analyzed the performance of the parallel implementation.

Qian Haichun has reviewed the entire parallel implementation work. He has contributed towards extracting video summarization information like the key frames from the MPEG video. He also spent time to perform the final integration and debugging for efficient working of the system.